Dynamic Functional-Structural Plant Modelling approach to investigate the consistency of ground coverage ratio to predict agrivoltaic systems yields

Corentin Coutellier1, Rémi Vézy2, Ya Brigitte Assoa1, Angélique Christophe3, Didier Combes4, Stéphane Guillerez1, Christophe Ménézo5

1 Univ. Grenoble Alpes, CEA, Liten, Campus Ines, F-73375 Le Bourget-du-Lac, France.

2 UMR AMAP, Cirad, Montpellier, France. Corresponding author: Remi.Vezy@cirad.fr

3 LEPSE, INRAE, Montpellier, France.

4 INRAE – UR4 – URP3F, Le Chêne RD 150, BP 6, F-86600 Lusignan, France.

5 Univ. Savoie Mont-Blanc, LOCIE UMR CNRS 5271, Campus Scientifique Savoie Technolac, F-73376 Le Bourget-du-Lac, France.

Keywords: virtual plant, agrivoltaic efficiency, photovoltaic, KPIs, modelling

Welcome to the 11th International Conference on Functional-Structural Plant Models (FSPM2026), taking place in Montpellier, France, October 12-16, 2026. FSPM2026 aims to gather scientists working in the multi-disciplinary field of plant modelling and its applications.

The conference will highlight a broad spectrum of topics at the intersection of plant science, modelling, and technology.

This document serves as a template and provides guidelines:

Abstract Guideline

Dear authors, when submitting your abstract you are kindly requested to

- use this template

- provide no more than two pages

- underscore presenting author and add one email-address for the corresponding author

- text: maximal 5000 characters including blanks

- text font: Arial 11 pt in justified text

- up to five keywords that are different from the title

- do not change the page margins, header and footer size and/or font

- use the following citation styles within the text: (Kahlen and Chen, 2015) (Schmidt et al., 2019)

- use S.I. units and compound units, e.g. in the following form g m-2 or flowers per plant

- type Latin biological names in italic, e.g. Cucumis sativus L.

- you can structure the abstract using chapter names such as Introduction, Materials and Methods, Results and Discussion, Conclusion

- add a reference list of up to 5 references, displayed in the style shown at the end of this template

- you may include a figure of maximum size half of a page (but keeping margins) of an appropriate resolution with centered figure caption in 8 pt

Le message : STICS n‘est pas suffisant pour la modélisation de systèmes AgriPV.

Introduction : Introduire le PCR et son utilisation dans la réglementation.

Problématique : Parler de la problématique de la 3D + dynamique => solution ArchiCrop. La dynamique est importante à prendre en compte en raison de la sensibilité au stress des plantes selon leur stade phénologique (gel = premiers stades de développement, stress thermique pendant l‘anthèse...). On veut de la 3D pour prendre en compte l‘hétérogénéité spatiale induite par les panneaux sur le rayonnement (papier Laura, sur les bilans d‘énergies).

Hypothèse : Le PCR n‘est pas un bon indicateur pour décrire l‘impact du PV sur les cultures.

Objectif 1, montrer différentes configurations (selon ce qui se fait sur le marché, avec différents PCR). => Pas de relation linéaire entre le PCR et le rayonnement transmis, photosynthèse.

Avec un PCR légal fixe (40 % en France) mais d‘autres indicateurs variables (ex: Shading factor). Objectif 2, montrer que même à PCR constant on peut avoir une variabilité de résultats rayonnement, photosynthèse, température, transpiration.

Méthodologie : Utilisation de STICS + ArchiCrop pour générer des maquettes réalistes de céréales. Pour cet exemple on montre le blé.

- Sélectionner des grands design récurents

- Fixer le Photovoltaic Coverage Ratio en faisant varier le Shading Factor :

Avoir pleins de configurations différentes : Rayonnement, bilan énergétique, photosynthèse

Résultats :

- Une figure avec de la 3D et le rayonnement transmis (2 bandes, 1 avec les plantes pour la photosynthèse et l‘autre sans les plantes pour le rayonnement).

Introduction

Many policies are currently focusing on increasing the share of renewable energy in the global energy mix to reach sustainability targets. In this context, agrivoltaism has emerged as a promising solution to optimize the land-use efficiency by combining crop production and photovoltaic energy generation on the same land-area (Dupraz et al., 2011). However, although agrivoltaic systems can improve land-use efficiency, often quantified through the land equivalent ration (LER), photovoltaic installations above crops may also reduce crop yield.

To limit this risk, some regulations define a maximum ground coverage ratio (GCR) for agrivoltaic installations, which represents the vertical projection of photovoltaic panels onto the ground. Yet, it may not be sufficient to predict crop performance, because installations with the same GCR can generate different shading patterns depending on panel layout and tilt angle. As a result, other indicators such as the shading factor (SF) or the light homogeneity index (LHI) are sometimes proposed as alternative to better capture the radiative environment experienced by the crop.

However, SF and LHI remain indirect indicators solely based on radiation, and do not account for downstream processes that are highly non-linear and impacting crop physiological responses and yield formation, and possibly differently depending on the crop plant developmental stage.

Therefore, assessing whether differences in SF and LHI at a given GCR translate into differences in crop performance requires plant growth and functioning to be simulated jointly with the radiative environment. In addition, the evolving 3D structure of the crop canopy can itself alter light distribution within the stand over time, especially in tall crops such as cereals or maize, which further supports the need for dynamic simulations over the full crop cycle. In addition, the evolving 3D structure of the crop canopy can itself modify light distribution within the stand over time, especially in tall crops such as cereals or maize, which further supports the need for dynamic simulations over the full crop cycle.

In this work, we use a dynamic modelling approach to jointly simulate canopy development, light interception, photosynthesis, and energy balance in agrivoltaic systems. We test the hypothesis that neither GCR nor radiation-based indicators of light distribution alone can reliably predict crop performance, and that a dynamic crop or plant model is needed to capture the complex interactions governing these systems.

Material and Methods

To investigate the hypothesis that GCR alone is not sufficient to predict the performance of an agrivoltaic system, we fixed GCR at 40 %, corresponding to the maximum value allowed under French regulations, and frequently adopted by energy suppliers for fixed agrivoltaics installations. Using the fixed GCR, together with panel tilt angles representative of commercially available systems, we modelled six fixed agrivoltaic configurations at the location of Montpellier (43.610°N, 3.878°E).

For each configuration, we computed the total photosynthetically active radiation (PAR) reaching the ground without plants over a whole year from January to December 2025, and used these simulations to calculate SF and LHI. Two configurations had horizontal panels (0° tilt), one with North-South (NS) row orientation and one with East-West (EW) row orientations. The four remaining configurations had a tilt angle of 39°, corresponding to the optimum tilt for south-facing photovoltaic installations in  Montpellier according to PVGIS (source?)), and were oriented toward the North, West, South and East.

We used the STICS soil-crop model (source?) to simulate crop development. ArchiCrop (source?) was used to generate daily 3D plant mock-ups from botanical rules and dynamic constrains at the crop scale coming from the crop model (i.e. leaf area index and crop height). We then used ArchimedLight to simulate the 3D radiation interception of PAR and NIR, and PlantBiophysics (source?) to compute photosynthesis, leaf temperature and evapotranspiration at an hourly rate.

Results and Discussion

Bla

To simulate crop growth, we made the simplifying assumption that crop development was spatially uniform, despite the heterogeneity in incident radiation generated by the panel layouts.

Conclusion

Bla

Figure #: Homepage of the FSPM2026 conference webpage

Acknowledgments

This template is adapted from the FSPM2023 conference template.

References

Dupraz et al., 2011, Renewable Energy 36:2725-2732, doi: 10.1016/j.renene.2011.03.005

Kahlen and Chen, 2015, Front. Plant Sci. 6:1130, doi: 10.3389/fpls.2015.01130

Schmidt et al., 2019, Agronomy 1-29, doi: 10.3390/agronomy9080426